SECTION 01 66 00

# PRODUCT DELIVERY, STORAGE AND HANDLING

- GENERAL

RELATED DOCUMENTS AND PROVISIONS

All Contract Documents should be reviewed for applicable provisions related to the provisions in this document, including without limitation:

General Conditions, including, without limitation, Site Access, Conditions and Requirements;

Special Conditions.

PRODUCTS

Products are as defined in the General Conditions.

Contractor shall not use and/or reuse materials and/or equipment removed from existing Premises, except as specifically permitted by the Contract Documents.

Contractor shall provide interchangeable components of the same manufacturer, for similar components.

TRANSPORTATION AND HANDLING

Contractor shall transport and handle Products in accordance with manufacturer's instructions.

Contractor shall promptly inspect shipments to confirm that Products comply with requirements, quantities are correct, and products are undamaged.

Contractor shall provide equipment and personnel to handle Products by methods to prevent soiling, disfigurement, or damage.

STORAGE AND PROTECTION

Contractor shall store and protect Products in accordance with manufacturer's instructions, with seals and labels intact and legible.  Contractor shall store sensitive products in weather-tight, climate controlled enclosures.

For exterior storage of fabricated Products, Contractor shall place on sloped supports, above ground.

Contractor shall provide off-site storage and protection when Site does not permit on-site storage or protection.

Contractor shall cover products subject to deterioration with impervious sheet covering and provide ventilation to avoid condensation.

Contractor shall store loose granular materials on solid flat surfaces in a well-drained area and prevent mixing with foreign matter.

Contractor shall provide equipment and personnel to store Products by methods to prevent soiling, disfigurement, or damage.

Contractor shall arrange storage of Products to permit access for inspection and periodically inspect to assure Products are undamaged and are maintained under specified conditions.

– PRODUCTS Not Used.

– EXECUTION Not Used.

END OF DOCUMENT